SAFETY  DATA  SHEET

SECTION  I   -  PRODUCT  IDENTIFICATION

PRODUCT  NAME:	CD 4250 PART B				              PRODUCT  CODE:	CD4250-B

PRODUCT  CLASS:	Polyol					              CAS  NUMBER:	None

| COMPANY NAME: | MANUFACTURED FOR: Coyote Quick Adhesive CD4250 B 419 N. Curtis Rd. Boise, Idaho, USA 83706 Telephone no.:208-429-0026 www.coyotedesign.com |  |  |
| --- | --- | --- | --- |

EMERGENCY ASSISTANCE:

CHEMTREC® (24 hours) U.S. : 1-800-424-9300 International: +1-703-527-388

SECTION  II  -  HAZARDS  IDENTIFICATION

Information In accordance with 29 CFR 1910.1200

GHS Classification:

Skin Irritation Category 2

Eye irritation Category 2A

Acute Toxicity Inhalation Category 4

GHS Label elements:

Pictograms

Signal Word: Warning

Hazard Statements:

H315:  Causes Skin Irritation

H320: Causes eye irritation

H335: May cause respiratory Irritation

Precautionary Statements:

P261: Do not breathe  vapors, mist or spray

P264: Wash thoroughly hands after handling

P280: Wear eye and face protection. Wear protective gloves

P301+P330+P331: If swallowed: Rinse mouth. Do not induce vomiting.

P302+P352: If on Skin: Wash with plenty of soap and water

P305+P351+P388: If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy         to do so. Continue rinsing.

P332+P313: If skin irritation occurs :Get medical attention

P337+P313: If eye irritation persists: Get medical attention

P342+P311: If experiencing respiratory symptoms: Call a poison center or physician

P404: Store in a well-ventilated place

P232: Protect from moisture

P233: Keep container tightly closed

P501: Dispose of contents and /or container according to Federal State/Provincial and local governmental regulations.

SECTION  III  -  COMPOSITION  /  INFORMATION  ON  INGREDIENTS

| COMPONENT | CAS NO. | Weight | ACGIH TLV | OSHA PEL |
| --- | --- | --- | --- | --- |
| NJTSRN-CD4250-B | -- | -- | -- | -- |

SECTION  IV  -  FIRST  AID  MEASURES

SKIN  CONTACT :	Remove contaminated clothing or shoes, wipe excess from skin and flush with

plenty of water for at least 15 minutes.  Use soap if available or follow by washing

with soap and water. Do not reuse clothing until thoroughly cleaned.

EYE  CONTACT :	Flush eyes with plenty of water for at least 15 minutes while holding eyelids open

and seek medical attention.

INHALATION :	Remove victim to fresh air and provide oxygen if breathing is difficult. Give artificial respiration if not breathing and seek medical help immediately.

INGESTION :	Do not give any liquids (do not induce vomiting) if victim is unconscious or very drowsy. Get medical help immediately.

SECTION  V  -  FIRE  FIGHTING  MEASURES

EXTINGUISHING  MEDIA:	Use Dry Chemical,  Carbon Dioxide,  Foam  or  Water Fog

SPECIAL  FIRE  FIGHTING  PROCEDURES  AND  PRECAUTIONS :

Full emergency equipment with self-contained breathing apparatus and full protective clothing should be worn by fire fighters. Do not enter a confined space without full bunker gear, including a positive pressure NIOSH approved self-contained breathing apparatus.  During fire, irritating and toxic gases may be generated by thermal decomposition or combustion.

SECTION  VI  -  ACCIDENTAL  RELEASE  MEASURES

SPILL  OR  LEAK  PROCEDURES:

Remove all sources of ignition and ventilate the area. Dike and contain spilled material and control further

spillage if feasible.  Cover spill with clay, sand, saw dust, vermiculite, Fuller’s earth or other suitable absorbent. Collect material in non-leaking containers and seal tightly for disposal.     Refer to section 13 for disposal information.

SECTION  VII  -  HANDLING  AND  STORAGE

Handle in accordance with good industrial hygiene and safety practices. These practices include avoiding unnecessary exposure and removal of the material from eyes, skin and clothing. Employee education and training in the safe use and handling of this material are required under the OSHA Hazard communication standard.  Use with adequate ventilation.

STORAGE:	Store indoors in a dry place away from heat, sparks and flame. Keep containers tightly closed when not in use.

SECTION  VIII  -  EXPOSURE  CONTROLS  /  PERSONAL  PROTECTION

EYE  PROTECTION:	Safety glasses, splash goggles

SKIN  PROTECTION:	Avoid contact with skin and clothing.   Use chemical resistant protective gloves.

RESPIRATORY  PROTECTION:	Avoid breathing vapors which may be produced under some conditions such as heating.  Avoid breathing aerosols and mists. Use NIOSH / MSHA approved respiratory protection equipment when airborne exposure is excessive.

VENTILATION :	Hazard control from vapor or spray mist is ideally performed by the use of engineering controls. General or local ventilation or isolation  may prove adequate to keep airborne exposures below  exposure limits.

SECTION  IX  -  PHYSICAL  AND  CHEMICAL  PROPERTIES

PHYSICAL  FORM:	  Liquid

COLOR:			  Black

BOILING  POINT:	  Not available

SOLUBILITY:		  Negligible (in water)

WEIGHT  PER  GALLON:  10.8  pounds

VAPOR  DENSITY:	  Heavier than air

EVAPORATION  RATE :	  Not available

(N-Butyl Acetate)

LIQUID DENSITY:	  Higher than water

FLASH  POINT :		  >325 F (PMCC)

FLAMMABILITY  LIMITS:  UEL % Not established  												  LEL % Not established

AUTO  IGNITION  TEMP. :Not established

Note:  These physical data are typical values based on material tested but may vary from sample to sample.  Typical values should not be construed as a guarantee analysis of any specific lot or as specifications for the product.

SECTION  X  -  STABILITY  AND  REACTIVITY

STABILITY:				Product is stable under normal conditions of storage and handling.

CONDITIONS  TO  AVOID :		Moisture and high temperature

HAZARDOUS  POLYMERIZATION :	Will not occur under normal storage conditions.

DECOMPOSITION  PRODUCTS :	By heat and fire: Carbon dioxide, carbon monoxide, Aldehydes, acids and other organic substances may be formed.

SECTION  XI  -  TOXICOLOGICAL  INFORMATION

Product is not toxic

SECTION  XII  -  ECOLOGICAL  INFORMATION

Data not available.

SECTION  XIII  -  DISPOSAL  CONSIDERATIONS

Waste must be disposed of in accordance with federal, state and local environmental control regulations.  Incineration is the preferred method. Empty containers must be handled with care due to product residue. Do not heat or cut empty containers with electric or gas torch.

SECTION  XIV  -  TRANSPORTATION  INFORMATION

PROPER  SHIPPING  NAME :	Not Regulated by D.O.T.

HAZARD  CLASS or DIVISION:	None

UN / NA Number :		N/A

PACKING GROUP :		PG III

DOT  PRODUCT  RQ, Lb.	None

HAZARD  LABEL(S) :		None

HAZARD  PLACARD(S) :		None

SECTION  XV  -  REGULATORY  INFORMATION

TSCA  STATUS:			All ingredients in this products are listed in the T.S.C.A. inventory.

CERCLA  REPORTABLE  QUANTITY:	None

SARA  TITLE  III:

SECTION  302 EXTREMELY  HAZARDOUS  SUBSTANCE :		None

SECTION  311/312  HAZARD  CATEGORIES:			None

SECTION  313 TOXIC  CHEMICALS :				None

RCRA :

If discarded in its purchased form,  this product would not be a hazardous waste by listing. However, under RCRA,

it is the responsibility of the product user to determine at the time of disposal, whether a material containing the

product or derived from the product should be classified as a hazardous waste. ( 40 CFR 261.20-24).

CALIFORNIA PROPOSITION 65:  To the best of our knowledge this product does not contain any list of chemicals, which the state of California has found to cause cancer, birth defects or other reproductive harm.

SECTION  XVI  -  OTHER  INFORMATION

HMIS  RATINGS:		Health		Flammability		Reactivity

1		        1			      0